Information Technology

LAB REPORT SUBMISSION COVER PAGE

MICROPROCESSORS

Introduction to emu8086

Objective:

Emu8086 combines an advanced source editor, assembler, disassembler, software emulator (Virtual PC) with debugger. The emulator runs programs like the real microprocessor in a step-by-step mode. It shows the contents of registers, memory, stack, variables and flags. All memory values can be investigated and edited easily. In addition, the instructions can be executed in a regular forward direction and also in reverse direction.

Emu8086’s visual interface is very easy to work with. You can watch registers, flags and memory values while your program executes. It also provides views on Arithmetic & Logical Unit (ALU) that show the internal work of the central processor unit (CPU).

The objective is to observe and examine the 8086 arithmetic, logical and program flow instructions set using the Emu8086 simulator.

General procedures

- Execute Emu8086 by selecting its icon from the start menu, or by running Emu8086.exe.

- Select code examples from "File" -> "Examples". All examples are heavily commented, so it’s a great learning tool.

- To run the code example in the emulator, click [emulate] button (or press F5).

- Click [single step] button (or press F8) to step through the code one instruction at a time, observing changes in registers and the emulator screen. Click [step back] button (or press F6) to see what happens when reversing those changes.

Task 1: Quick study of the simulator operations 1

- Click on the icon of Emu8086 to start the simulator. When the simulator starts, you should see the starting message “Loading” as shown in Figure 1.

Figure 1: Emu8086 Loading Page

- After the initial screen is displayed, you should click on the [quick start tutor] option to start 8086 assembler tutorials. A screen as shown in Figure 2 will appear. Read all the tutorials.

Figure 2: Emu8086 Tutorials

Task 2: Quick study of the simulator operations 2

Enter the code shown above into the simulator. Run the code by clicking the [emulate] button. This will open the “original source code” window and an “emulator window” showing registers, memory, [single step] and [run] buttons.

A) What are AL and BL?  lower (8-bit) of general registers AX and BX

B) Where are the instructions stored when the program is assembled?

in the emulator (virtual PC)

C) What is the binary code for the instruction MUL? ___F6______

D) Where is the result of the operation stored? _____AX__________

E) What is the result displayed in the simulator?

Binary ___1100 1000____ Hex ___C8_____ Decimal ___200______

G) What type of numbering system is being used in this simulator?  bin, oct, dec, hex

H) How are negative numbers indicated in this numbering system?  highest bit is 1

I) What is the purpose of the IP register?__pointer to next instruction_

Task 3: Load first program and analyze the operations

- Click on the “file” menu and then on “open”. Choose “1_sample.asm” file.

- “1_sample.asm” source code will be displayed in the window.

- Run the code by clicking the [emulate] button. This will open the “emulated source code” window and a window showing registers, memory, [single step] and [run] buttons.

- Click on [single step] button. Observe and record all the changes taking place every time you click this button. Examine the contents of registers and comprehend their effects in the source code window. After the program is executed completely, an “emulator screen” window will be displayed as shown in Figure 3. It shows the results of the program.

- Based on your analysis in the previous step, deduce the reasons of the changes taking place in the registers.

Figure 3: Emulator Screen

Task 4:  Traffic lights program

- Select the “traffic lights” program from "file" -> "examples".

- Understand the program by observing the program flow and interpreting the instructions in the source code.

- Run the code by clicking the [emulate] button. This will open the “emulated source code” window and a window showing registers, memory, [single step] and [run] buttons.

- Click on the [single step] button. Observe and record the changes taking place every time you click this button. Examine the contents of registers and their effects in the source code window. After the first OUT instruction is executed, a window showing the simulation of traffic lights will be displayed as shown in Figure 4.

- Based on your analysis in the previous step, deduce the reasons of the changes taking place in the registers.

Figure 4: Traffic lights window

Task 5:  Add/Subtract program

- Select the “add/substract” program from "file" -> "examples".

- Understand the program by observing the program flow and interpreting the instructions in the source code.

- Run the code by clicking the [emulate] button. This will open the “emulated source code” window and a window showing registers, memory, [single step] and [run] buttons.

- Click on the [single step] button. Observe and record the changes taking place every time you click this button. Examine the contents of registers and their effects in the source code window.

- Based on your analysis in the previous step, deduce the reasons of the changes taking place in the registers.

Laboratory Exercise 1

Objective:

Introduce the student to the emu8086 emulator and some of its basic functionality. Additionally, students will investigate the various registers of the 8086 microprocessor and the instructions set used to manipulate them.

Task 1. Basic arithmetic

- Start emu8086. Close the welcome window. The main window opens.

- Open Base Converter by selecting “convertor”. It allows you to convert numbers from any system and to any system. You can work both with 8 bit and 16 bit values.

- Open Expression Evaluator by selecting “calculator”. It can be used to make calculations between numbers in different systems and convert numbers from one system to another.

- Choose “New” and specify Choose “New” and specify “BIN template” in emu8086.  Click OK.

- Replacing all but the “#make_BIN#” directive, type in the following:

- Compile and load the previous code into the emulator (press F5)

- Start emulation by clicking the “emulate” button on the toolbar. A new emulator window will appear.

- Single-step the program codes by pressing the “single step” button on the toolbar of the emulator window.

- Each time after pressing the “single step” button, check and record down the contents of AL, BL and CL registers and flags (“flags” button) in the table below. Use the first row (instruction) as example:

| Instruction | AL | BL | CL | Flags |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | CF | ZF | SF | OF | PF | AF | IF | DF |
| mov al,00000101b | 05h |  |  |  |  |  |  |  |  | 1 |  |
| mov bl,0ah | 05h | 0ah |  |  |  |  |  |  |  | 1 |  |
| mov cl,10o | 05h | 0ah | 08h |  |  |  |  |  |  | 1 |  |
| add al,bl | 0fh | 0ah | 08h |  |  |  |  | 1 |  | 1 |  |
| sub al,cl | 07h | 0ah | 08h |  |  |  |  |  |  | 1 |  |

Task 2.  More arithmetic-logical operations

Objective:

Using the previous code as a guide, write a simple program:

Step 1	Load 255 into the AL register

Step 2	Load 1 into the BL register

Step 3	Load 2 into the CL register

Step 4	Add BL to AL and store the result in AL

Step 5	Subtract CL from AL and store the result in AL.

- Single-step the program codes by pressing the “single step” button on the toolbar of the emulator window.

- Each time after pressing the “single step” button, check and record down the contents of AL, BL and CL registers and flags (“flags” button) in the table below. Use the first row (instruction) as example:

| Instruction | AL | BL | CL | Flags |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | CF | ZF | SF | OF | PF | AF | IF | DF |
| mov al,255 | ffh |  |  |  |  |  |  |  |  | 1 |  |
| mov bl,1 | ffh | 01h |  |  |  |  |  |  |  | 1 |  |
| mov cl,2 | ffh | 01h | 02h |  |  |  |  |  |  | 1 |  |
| add al,bl | 00h | 01h | 02h | 1 | 1 |  |  | 1 | 1 | 1 |  |
| sub al,cl | feh | 01h | 02h | 1 |  | 1 |  |  | 1 | 1 |  |

Task 3.  More logical operations

Objective:

Flags Register - determines the current state of the processor.

The flags are modified automatically by CPU after mathematical operations, this allows to determine the type of the result, and to determine conditions to transfer control to other parts of the program. Generally you cannot access these registers directly.

- Carry Flag (CF) - this flag is set to 1 when there is an unsigned overflow. For example when you add bytes 255 + 1 (result is not in range 0...255). When there is no overflow this flag is set to 0.

- Parity Flag (PF) - this flag is set to 1 when there is even number of one bits in result, and to 0 when there is odd number of one bits.

- Auxiliary Flag (AF) - set to 1 when there is an unsigned overflow for low nibble (4 bits).

- Zero Flag (ZF) - set to 1 when result is zero. For non-zero result this flag is set to 0.

- Sign Flag (SF) - set to 1 when result is negative. When result is positive it is set to 0. (This flag takes the value of the most significant bit.)

- Trap Flag (TF) - Used for on-chip debugging.

- Interrupt enable Flag (IF) - when this flag is set to 1 CPU reacts to interrupts from external devices.

- Direction Flag (DF) - this flag is used by some instructions to process data chains, when this flag is set to 0 - the processing is done forward, when this flag is set to 1 the processing is done backward.

- Overflow Flag (OF) - set to 1 when there is a signed overflow. For example, when you add bytes 100 + 50 (result is not in range -128...127).

Using the previous code as a guide, write the following program.

Then emulate and single-step it.

For each of the mathematical comparisons listed in the code segments, write down the expression, all flags influenced by the comparison, and very briefly explain each of their meaning.

In other words, explain why a given how a given expression is related to the flags set, or reset.

Note! The NOP operation would be replaced with a proper conditional jump operation in real programming.

Fill the table below (type the conditional jump instruction mnemonic, that is required to accomplish the comparison and the conditional jump, and record the flag values after each NOP operation).  Is the condition true or false? 

Use the block A (first comparison code segment) solution as example:

| Block | Conditional Jump |  |  | Flags |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Opcode | Description | True/False | CF | ZF | SF | OF | PF | AF | IF | DF |
| A | JE | Jump if both operands equal | True | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 0 |
| B | JA | Jump if first operand is Above second operand | T |  |  |  |  |  |  | 1 |  |
| C | JG | Jump if first operand is Greater then second operand | T | 1 |  |  |  | 1 | 1 | 1 |  |
| D | JB | Jump if first operand is Below second operand | T | 1 |  |  |  | 1 | 1 | 1 |  |
| E | JL | Jump if first operand is Less then second operand | T | 1 |  | 1 |  | 1 | 1 | 1 |  |
| F | JG | Jump if first operand is Greater then second operand | T |  |  |  |  |  |  | 1 |  |
| G | JA | Jump if first operand is Above second operand | T |  |  | 1 |  |  |  | 1 |  |

Laboratory Exercise 2

Task 1. Writing and Running Assembly Code in Emu8086

- Start emu8086. Close the welcome window. The main window opens.

- Choose “New” and specify “COM template” in emu8086.  Click OK.

- Enter the following program to the editor. The program waits for a keystroke and then returns to the OS.

- Start the emulation of the program by clicking the “emulate” button on the toolbar. The original source and emulator windows open.

- Run the program codes by pressing the “run” button on the toolbar of the emulator window.

- The emulated DOS window opens.

- Take a screen capture (e.g. using Snipping Tool) and attach it below:

- Attach the screen capture here

- Press any key. The info window opens. Click OK.

Task 2. Writing and Running Assembly Code in Emu8086

- Start emu8086. Close the welcome window. The main window opens.

- Choose “New” and specify Choose “New” and specify “COM template” in emu8086.  Click OK.

- Enter the following program to the editor. (This is the classic Hello World!)

- Start the emulation of the program by clicking the “emulate” button on the toolbar. The original source and emulator windows open.

- Click the”run” button on the toolbar. The text “Hello World!” appears on the emulated DOS window (emulator screen).

- Take a screen capture (e.g. using Snipping Tool) and attach it below:

- Attach the screen capture here

- Close the emulator window.

Task 3. A simple arithmetic program

Your task is to write a program that calculates the following formula:

(3010+1510)*(57510-22510)+21010

- Choose “New” and specify “COM template” in emu8086. Answer No to the question about saving changes of the current file. Click OK.

- Enter the following code to the editor:

- Start emulation by clicking the “emulate” button on the toolbar. A new emulator window will appear.

- Single-step the program codes by pressing the “single step” button on the toolbar of the emulator window.

- Each time after pressing the “single step” button, check and record down the contents of AX and BX registers in the table below.

| Instruction | AX | BX |
| --- | --- | --- |
| mov ax,30 | 1eh |  |
| add ax,15 | 2dh |  |
| mov bx,575 | 2dh | 023fh |
| sub bx,225 | 2dh | 015eh |
| mul bx | 3d86h | 015eh |
| add ax,210 | 3e58h | 015eh |

- Using a calculator, calculate the answer for the above arithmetic operations here .....15 960 (3e58h).......

- Is it the same as the final answer in the AX register? .....yes.......

Task 4.  Another simple arithmetic program

In this example, we will develop a program to perform the following arithmetic operations:

1210*(20010-22510)+12710

Procedure:

- Develop an assembly language program to perform the above arithmetic operations by only using registers AX and BX (write the program in the table below).

- Write the program into the assembler editor in emu8086

- Emulate the program by pressing the “emulate” button on the toolbar.

- In the emulator window, single-step the program codes by pressing the “single step” button on the toolbar.

- Each time after single -stepping, observe and record down the contents of AX and BX registers in the table below.

| Instruction | AX | BX |
| --- | --- | --- |
| mov ax,12 | 0ch |  |
| mov bx,200 | 0ch | c8h |
| sub bx,225 | 0ch | ffe7h |
| mul bx | fed4h | ffe7h |
| add ax,127 | ff53h | ffe7h |

- Using a calculator, calculate the answer for the above arithmetic operations: .........-173 (ff53h)........

- Is it the same as the final answer in the AX register? ..............yes.........................

Laboratory Exercise 3

Task 1. Writing an 8086 Assembly Language Program to find square and cube of a number

- Start emu8086. Choose “New” and specify “COM template” in emu8086.

- Enter the following code to the editor:

; Program to find square and cube of a number

ORG 	100h

JMP 	START	; Jump to the beginning of the code

X   	DW 	04H      	;

SQ  	DW 	?        	;

CB  	DW 	?        	;

START:   	MOV 	AX,X    	;

MOV 	BX,X    	;

MUL 	BX      	;

MOV 	SQ,AX   	;

MUL 	BX      	;

MOV 	CB,AX   	;

MOV 	AH,4CH  	;

INT 	21H     	;

RET

- Emulate the program by pressing the “emulate” button on the toolbar.

- In the emulator window, single-step the program codes by pressing the “single step” button on the toolbar.

- Describe here, how the square and cube are calculated.
square is number x number
cube is square x square

- Add the missing comments in the code after the semicolons (;).

; Program to find square and cube of a number

ORG 	100h

JMP 	START	; Jump to the beginning of the code

X   	DW 	04H      	;declare variable X and set 04H

SQ  	DW 	?        	;declare variable SQ not initialized

CB  	DW 	?        	;declare variable CB not initialized

START:   	MOV 	AX,X    	;load value of X to AX register

MOV 	BX,X    	;load value of X to BX register

MUL 	BX      	;multiply AX and BX

MOV 	SQ,AX   	;move value of AX to SQ variable

MUL 	BX      	;multiply AX and BX

MOV 	CB,AX   	;move value of AX to CB variable

MOV 	AH,4CH  	;load terminate code to AH register

INT 	21H     	;activates interupt procedure (DOS)

RET

Laboratory Exercise 4

Task 1. Character input and output

Your task is to create program that reads characters from keyboard and displays on the screen. The program exits when the Enter key is pressed (the ASCII code of enter is 0dh).

The function to input a character with echo is:

mov	ah,1	; keyboard input subprogram

int	21h	; call ms-dos input character

;the character input is stored in al

The function to display a character on screen is:

mov	dl,character	;dl=ascii code of the character, e.g. ‘a’ or 0dh

mov	ah,2	;character output subprogram

int	21h	;call ms-dos output character

The function to terminate a program and return to ms-dos is:

mov	ax,4c00h	;code for return to ms-dos
int	21h	;call ms-dos terminate program

The flowchart of the program is below.

- Start emu8086. Choose “New” and specify “COM template” in emu8086.

- Write the program code below (Hint! Set the editor into Replace mode when entering the code).

;                                                           Write your code here!

; Program to input characters from keyboard

; Code 0dh (Enter) exits to the OS

ORG 	100h

START:  mov	ah,1	 ; keyboard input subprogram

int	21h	 ;call ms-dos input character

cmp al,0dh 	 ;compare to Enter ascii code

je EXIT     		;if equal jump to exit

;else print character

mov	dl,al	;dl=ascii code of the character, e.g. ‘a’ or 0dh

mov	ah,2	;character output subprogram

int	21h	;call ms-dos output character

jmp START  	 ;jump back to start

EXIT:  	MOV 	AH,4CH  	;code for return to ms-dos

INT 	21H     		;ms-dos procedure

RET

- Enter the code of the program to the editor:

- Emulate the program by pressing the “emulate” button on the toolbar.

- In the emulator window, run the program by pressing the “run” button on the toolbar.

- Type characters and then enter to exit (Note that the input function echoes the character).

- Take a screen capture (e.g. using Snipping Tool) and attach it below:

- Attach the screen capture here

- Click OK.

Laboratory Exercise 5

Task 1. User-defined subroutines

Your task is to create a program that uses subroutines to input and output a character, to output a string and to exit the program. The program uses ready-made subroutines that are described at the end of this exercise.

The flowchart of the program is shown below.

Complement the following program:

- Add the missing instructions at ‘Enter your code here’ (Hint! Set the editor into Replace mode when entering the code):

- Write a string (message): “Anna merkki (Enter lopettaa)”

- Read a character from keyboard

- Write the character on the screen

- Exit the program if Enter (0dh) was pressed.

;START-OF-PROGRAM

org 	100h

jmp 	start

msg 	db  	'Enter a character (the enter key ends)',0dh,0ah,'$'

start:

lea  	dx,msg 	; load string address to dx

call 	puts   	; output string

;      Write your code here!

msg2 	db  	'Anna merkki (Enter lopettaa)',0dh,0ah,'$'

lea  	dx,msg2	; load string address to dx

call 	puts   	; output string

read:   	call    getcb  	;read character (without echo)

cmp   al,0dh  	;compare input to Enter

je        exit   	;jump to exit if equal

mov     dl,al

call       putc    	;write character

jmp     read   	;jump to reading loop

exit:   	call    exitus  	;terminate program

ret

; *****************************************************************

;                User-defined subroutines

;        Always paste these at the end of your program

; *****************************************************************

;***** puts

;The subroutine to display a string terminated by '$'

;dx contains address of string

puts:  mov	ah,9	; output string subprogram

int	21h	; call ms-dos output string

ret		;

;***** gets

;The subroutine to input a character with echo

getc:  mov	ah,1; keyboard input subprogram

int	21h	; call ms-dos input character

ret		; the character input is stored in al

;***** getcb

;The subroutine to input a character without echo

getcb: mov	ah,8	; keyboard input subprogram

int	21h	; call ms-dos input character

ret		; the character input is stored in al

;***** putc

;The subroutine to display a character on screen

;dl contains ascii code of the character, e.g. 'a' or 0dh

putc:  mov	ah,2	;character output subprogram

int	21h	;call ms-dos output character

ret

;***** exitus

;The subroutine to terminate a program and return to ms-dos

exitus: mov	ax,4c00h	;code for return to ms-dos

int	21h	;call ms-dos terminate program

ret

;******************* End-of-subroutines *****************************

ret

;END-OF-PROGRAM

- Start emu8086. Choose “New” and specify “COM template” in emu8086.

- Enter the program to the editor.

- Emulate the program by pressing the “emulate” button on the toolbar.

- In the emulator window, run the program by pressing the “run” button on the toolbar.

- Take a screen capture (e.g. using Snipping Tool) and attach it below:

- Attach the screen capture here

Laboratory Exercise 6

Task 1. Writing a arithmetic program

By now you should have acquired all the necessary knowledge to design your own program. Thus, in this exercise you are required to construct your own program to perform arithmetic operations based on the last four digit of your student ID.

Assume that the last four digits of your student ID are abcd (e.g. 1234). You are required to perform the following arithmetic operation:

2 * (ab10 + cd10) * (510 + ab10) – 510

Your final answer should be stored in the AX register.

;START-OF-PROGRAM

org 	100h

jmp 	start

; define your variables below

msg    db  'Enter 4 digit number:',0dh,0ah,'$'

ab       dw  0       ;declare variable for first 2 numbers

cd       dw  0       ;declare variable for last 2 numbers

start:

lea  	dx,msg 	; load string address to dx

call 	puts   	; output string

;      Write your code here!

;Read 4 digit number

mov     bx,10

call    	getcb   	;read 1. digit

mov    	ah,0    	;reset ah reg

sub     	al,30h   	;sub ascii code of 0

mul     	bx     	;multiply 1. digit by 10

mov     	ab, ax  	;store 1.digit in ab

call    	getcb  	;read 2.digit

mov     	ah,0    	;reset ah reg

sub     	al,30h  	;sub ascii code of 0

add    	ab,ax   	;add 2.digit to ab

call    	getcb   	;read 3. digit

mov     	ah,0    	;reset ah reg

sub     	al,30h  	;sub ascii code of 0

mul     	bx     	;multiply 3. digit by 10

mov     	cd, ax  	;store 3.digit in ab

call    	getcb   	;read 4.digit

mov     	ah,0   	;reset ah reg

sub     	al,30h   	;sub ascii code of 0

add     	cd,ax  	;add 4.digit to ab

;calc

mov 	ax,ab

add 	ax,cd   ;ab+cd

mov 	bx,ab

add 	bx,5    ;5+ab

mul 	bx      ;(ab+cd)*(5+ab)

mov 	bx,2

mul 	bx      ;*2

sub 	ax,5    ;-5

call    	exitus  ;terminate program

ret

; *****************************************************************

;                User-defined subroutines

;        Always paste these at the end of your program

; *****************************************************************

;***** puts

;The subroutine to display a string terminated by '$'

;dx contains address of string

puts:  mov	ah,9	; output string subprogram

int	21h	; call ms-dos output string

ret		;

;***** gets

;The subroutine to input a character with echo

getc:  mov	ah,1; keyboard input subprogram

int	21h	; call ms-dos input character

ret		; the character input is stored in al

;***** getcb

;The subroutine to input a character without echo

getcb: mov	ah,8	; keyboard input subprogram

int	21h	; call ms-dos input character

ret		; the character input is stored in al

;***** putc

;The subroutine to display a character on screen

;dl contains ascii code of the character, e.g. 'a' or 0dh

putc:  mov	ah,2	;character output subprogram

int	21h	;call ms-dos output character

ret

;***** exitus

;The subroutine to terminate a program and return to ms-dos

exitus: mov	ax,4c00h	;code for return to ms-dos

int	21h	;call ms-dos terminate program

ret

;******************* End-of-subroutines *****************************

ret

; END-OF-PROGRAM

- Click on “com template” from “file” -> “new”.

- Edit your source code in the template.

- Emulate the program and single-step the code. Observe and record the contents of the registers each time after single-stepping. Deduce the reasons of the changes taking place in the registers.

- Calculate the answer for the arithmetic operations using a calculator and validate the final answer with the content of the AX register.

Laboratory Exercise 7

To make programming easier there are some common functions that can be included in your program. To make your program use functions defined in other file you should use the INCLUDE directive followed by a file name.

Compiler automatically searches for the file in the same folder where the source file is located, and if it cannot find the file there - it searches in Inc folder.

To use any of the functions in emu8086.inc you should have the following line in the beginning of your source file: include 'emu8086.inc'

Task 1. Product of two numbers - show result as unsigned integer

Write a program that reads two 16-bit signed integers (WORDs) between -128…127 from keyboard, calculates their products and prints the result in screen.

The following procedures from ‘emu8086.inc’ are used:

- PRINT_STRING: Procedure to print a null terminated string at current cursor position, receives address of string in DS:SI register.

- SCAN_NUM:  Procedure that gets the multi-digit SIGNED number from the keyboard, and stores the result in CX register.

- PRINT_NUM: Procedure that prints a signed number in AX register.

- Complement the following program: Add the missing instructions.

include "emu8086.inc"

; START-OF-PROGRAM

org 100h

jmp start

; Memory variables

First	dw	0

prompt1	db	0dh, 0ah,"Enter first integer (-128...127): ",0

prompt2	db	0dh, 0ah,"Enter second integer (-128...127): ",0

product	db	0dh, 0ah,"Product = ",0

interror1	db	0dh, 0ah, "Number is too big!", 0dh, 0ah,0

interror2	db	0dh, 0ah, "Number is too small!", 0dh, 0ah,0

Result  	db	0dh, 0ah, "The product is: ",0

START:

RD1:    lea	SI, prompt1	; Address of prompt1: “Enter first int”

call	PRINT_STRING ; Print prompt1

call	SCAN_NUM	; Read the first integer into CX

cmp     cx,127		; Is it too big (>127)?

jg      errG1		; print error

cmp     cx,-128		; Is it too small (<-128)?

jl      errL1		; print error

CONT:   mov First,cx		; Move the first number into ‘First’

RD2:	lea	SI, prompt2		; Address of prompt2: “Enter second int”

call	PRINT_STRING	; Print prompt2

call	SCAN_NUM		; Read the second integer into CX

cmp     cx,127		; Is it too big (>127)?

jg      errG2		; print error

cmp     cx,-128		; Is it too small (<-128)?

jl      errL2		; print error

CONT2:	mov ax,First	; Move the first number into AX

mul     cx		    ; Multiply

lea	    SI, Result	; Address of string: “The product is”

call	PRINT_STRING ; Print string

CALL	PRINT_NUM	; Print the result

bort:	mov	ax,4c00h	; code for return to ms-dos

int	21h	; call ms-dos terminate program

ret

errG1: lea	    SI,interror1    ; Address of error message 1

call	PRINT_STRING	; Print interror1

jmp     RD1     		; Read again (jump to RD1:)

errL1: lea	    SI,interror2    ; Address of error message 1

call	PRINT_STRING	; Print interror1

jmp     RD1     		; Read again (jump to RD1:)

errG2: lea	    SI,interror1    ; Address of error message 1

call	PRINT_STRING	; Print interror1

jmp     RD2     		; Read again (jump to RD1:)

errL2: lea	    SI,interror2    ; Address of error message 1

call	PRINT_STRING	; Print interror1

jmp     RD2     		; Read again (jump to RD1:)

; Macro definitions

DEFINE_PRINT_STRING

DEFINE_SCAN_NUM

DEFINE_PRINT_NUM

DEFINE_PRINT_NUM_UNS

end 					;END-OF-PROGRAM

- Enter the program to the editor.

- Save the program in C:\emu8086\MySource and give it name Lab7.asm.

- Emulate the program by pressing the “emulate” button on the toolbar.

- In the emulator window, run the program by pressing the “run” button on the toolbar.

- Take a screen capture (e.g. using Snipping Tool) and attach it below:

- Attach the screen capture here

Laboratory Exercise 8

The following procedures from ‘emu8086.inc’ are used:

- PRINT_STRING: Procedure to print a null terminated string at current cursor position, receives address of string in DS:SI register.

- SCAN_NUM:  Procedure that gets the multi-digit SIGNED number from the keyboard, and stores the result in CX register.

- PRINT_NUM: Procedure that prints a signed number in AX register.

- PUTC: PUTC char  - macro with 1 parameter, prints out an ASCII char at current cursor position.

Task 1. A program to generate the first 23 Fibonacci numbers

Write a program that generates the first 23 Fibonacci numbers and prints tem in screen.

In mathematics, the Fibonacci numbers or Fibonacci sequence are the numbers in the following integer sequence:

0, 1, 2, 3, 5, 8, 13, 21, 34, 55, 89, 144, …

By definition, the first two numbers in the Fibonacci sequence are 1 and 1, or 0 and 1, depending on the chosen starting point of the sequence, and each subsequent number is the sum of the previous two.

- Complement the following program: Add the missing instructions.

include "emu8086.inc"

; START-OF-PROGRAM

org 100h

jmp start

; Memory variables

f0	dw	0	; Previous number

f1	dw 	1	; Second etc. number

n	equ	22	; Loop count (22 first numbers)

prompt1	db	0dh, 0ah,"The first 23 Fibonacci numbers: ",0

start:	lea	SI, prompt1	; Address of prompt1: “The first...”

mov 	cx,22	; Loop counter

RD1:	call	PRINT_STRING ; Print prompt1

mov	ax,f0    	; First Fibonacci number to ax

call PRINT_NUM_UNS	; Print first number

PRINT ' '		; Print space character

;calculate next Fibonacchi number

up1:mov bx,f0		; LOAD f1 to BX

mov ax,f1		; Load f0 to AX

add bx,ax		; Add: BX=f1+f0

call PRINT_NUM_UNS		; PRINT it

PRINT ' '		; Print space character

mov f0,ax		; Update “Previous number”

mov f1,bx		; Update “Second number”

;inc cx		; Increment loop counter

loop up1		; Print next (jmp to up1:)

abort:	mov	ax,4c00h	; code for return to ms-dos

int	21h	; call ms-dos terminate program

ret

; Macro definitions

DEFINE_PRINT_STRING

DEFINE_PRINT_NUM

DEFINE_PRINT_NUM_UNS

end 					;END-OF-PROGRAM

- Enter the program to the editor.

- Save the program in C:\emu8086\MySource and give it name Lab8.asm.

- Emulate the program by pressing the “emulate” button on the toolbar.

- In the emulator window, run the program by pressing the “run” button on the toolbar.

- Take a screen capture (e.g. using Snipping Tool) and attach it below:

- Attach the screen capture here

- Why the maximum number of Fibonacci numbers to be shown with this program is 23?

owerflow of 16-bit register at 26th number

Laboratory Exercise 9

The following procedures from ‘emu8086.inc’ are used:

- PRINT_STRING: Procedure to print a null terminated string at current cursor position, receives address of string in DS:SI register.

- SCAN_NUM:  Procedure that gets the multi-digit SIGNED number from the keyboard, and stores the result in CX register.

- PRINT_NUM: Procedure that prints a signed number in AX register.

- GET_STRING:  Procedure to get a null terminated string from a user, the received string is written to buffer at DS:DI, buffer size should be in DX. Procedure stops the input when 'Enter' is pressed.

Task 1. A program to read and write a text string

Write a program that reads a text string into a memory buffer and then writes it back.

- Complement the following program. Add the missing instructions.

include "emu8086.inc"

; START-OF-PROGRAM

org 100h

jmp start

; Memory variables:

msg1	db	"Enter a string (max 128 characters): ",0

; the buffer to save the entered string

mystr	db  	128 dup (0),0

endl  	db  	0dh,0ah,0

start:  lea 	SI, msg1    	; Message address

CALL	PRINT_STRING	; Print message from [SI]

lea     DI,mystr  ; String mystring: Read string here!

mov     dx,128	; String length

call    GET_STRING	; Read string into [DI]

lea 	   si,endl	; cr+lf buffer address

call    PRINT_STRING	; Print cr+lf [SI]

lea 	SI, mystr	; String mystring buffer address

call	PRINT_STRING	; Print string from [SI]

abort:	mov	ax,4c00h	; code for return to ms-dos

int	21h	; call ms-dos terminate program

ret

; Macro definitions

DEFINE_GET_STRING

DEFINE_PRINT_STRING

DEFINE_PRINT_NUM

DEFINE_PRINT_NUM_UNS

end 					;END-OF-PROGRAM

- Enter the program to the editor.

- Save the program in C:\emu8086\MySource and give it name Lab9A.asm.

- Emulate the program by pressing the “emulate” button on the toolbar.

- In the emulator window, run the program by pressing the “run” button on the toolbar.

- Take a screen capture (e.g. using Snipping Tool) and attach it below:

- Attach the screen capture here

Task 2. Test if the string is a palindrome

Modify the previous program so, that it checks whether the string is a palindrome. Complement the following program. Add the missing instructions.

include "emu8086.inc"

; START-OF-PROGRAM

org 100h

jmp start

; Memory variables:

msg1	db	"Enter a string (max 128 characters): ",0

msg2	db 	0ah,0dh,"The string is NOT a palindrome.",0

msg3    db  	0ah,0dh,"The string is a palindrome.",0

; the buffer to save the entered string

mystr	db  	128 dup (0),0

mystrREV	db  	128 dup (0),0

endl  	db  	0dh,0ah,0

length	db	0

start:  lea 	SI, msg1    	; Message address

CALL	PRINT_STRING	; Print message from [SI]

lea     DI,mystr  	    ; String mystring: Read string here!

mov     dx,128	        ; String length

call    GET_STRING	    ; Read string into [DI]

lea 	si,endl	        ; cr+lf buffer address

call    PRINT_STRING    ; Print cr+lf [SI]

;lea 	SI, mystr	; String mystring buffer address

;call	PRINT_STRING	; Print string from [SI]

; count the number of characters in the buffer mystr into CX:

mov	cx,0	; start from 0

lea	SI,mystr	; Point SI to mystr

tess:	mov	al,[SI],0	; End of data?

cmp	al,0	; -“-

je	seur	; Proceed to next step

inc	cl	; Increment data counter

inc	SI	; Increment data pointer

jmp	tess	; Check next

; copy mystr into mystrREV in reverse order

seur:	mov	length,cl	; Store # of characters in length

lea si,mystr	; Source buffer address id SI(decremented)

lea	di,mystrREV	; Result buffer address into DI

add di,cx       ; move to last position

sub di,1

coop:		;mov al,[si]		; Copy character from source

;mov [di],al	    ; Copy character to destination

movsb           ; copy from si to di

sub di,2          ; decrement di (-2 because movsb increment both si and di)

loop coop    	; Take next if not done

; print both buffers

lea 	si,mystr

call	print_string	; Print mystr

lea 	si,endl

call	print_string	; Print cr+lf

lea 	si,mystrREV

call	print_string	; Print mystrREV

lea 	si,endl

call	print_string	;print cr+lf

; compare strings. If equal => palindrome

mov	cl,length	; # of characters in buffers

lea si,mystr	; address of first buffer

lea	di,mystrREV	; address of second buffer

niis:	cmp	cl,0	; test if end-of-comparison/buffer

je  positive	; jump to ok, palindrome/empty buffer

mov al,[si]	; Source buffer address

mov bl,[di]	; Result buffer address

cmp	al,bl   ; Are same, still chance?

jne negative	; Nop, jump to print NOT-message and exit

inc si	    ;increment source pointer

inc di  	; increment destination pointer

dec cl	    ; decrement counter

jmp niis	; Try next

positive: lea	SI,msg3	; Yess, palindrome

call	PRINT_STRING	; Print it

jmp	    abort	        ; and exit

negative: lea	si,msg2	; NOT a palindrome

call	PRINT_STRING	; Print it and exit

abort:	mov	ax,4c00h	; code for return to ms-dos

int	21h	; call ms-dos terminate program

ret

; Macro definitions

DEFINE_GET_STRING

DEFINE_PRINT_STRING

DEFINE_PRINT_NUM

DEFINE_PRINT_NUM_UNS

end 					;END-OF-PROGRAM

- Enter the program to the editor.

- Save the program in C:\emu8086\MySource and give it name Lab9B.asm.

- Emulate the program by pressing the “emulate” button on the toolbar.

- In the emulator window, run the program by pressing the “run” button on the toolbar. Take a screen capture from both cases  (e.g. using Snipping Tool) and attach then below:

                                          Attach the first screen capture here

Attach the second screen capture here